Infinity    (version1)    27 june 95

DMX PROTOCOL CHANNEL ASSIGNEMENT

| Channel | Function |
| --- | --- |
| 1 | COLOR |
| 2 | 1° GOBO WHEEL + EFFECTS WHEEL |
| 3 | 2° GOBO WHEEL |
| 4 | GOBO INDEX + ROTATION |
| 5 | IRIS |
| 6 | FOCUSING |
| 7 | DIMMER / SHUTTER |
| 8 | LAMP ON/OFF + SPECIAL FUNCTIONS |
| 9 | PAN |
| 10 | TILT |

CHANNEL  1  =  COLOUR

percent values

1  >  4 %   WHITE

5  >  8 %   WHITE / RED

9  > 12 %  RED

13 > 16 %  RED / YELLOW

17 > 20 %  YELLOW

21 > 24 %  YELLOW / MAGENTA

25 > 28 %  MAGENTA

29 > 32 %  MAGENTA / GREEN

33 > 36 %  GREEN

37 > 40 %  GREEN / ORANGE

41 > 44 %  ORANGE

45 > 48 %  ORANGE / BLUE

49 > 52 %  BLUE

53 > 56 %  BLUE / PINK

57 > 60 %  PINK

61 > 64 %  PINK / WHITE

65 > 68 %  SPIN 1 (   3 rpm)

69 > 72 %  SPIN 2 (   5 rpm)

73 > 76 %  SPIN 3 (   8 rpm)

77 > 80 %  SPIN 4 (  14 rpm)

81 > 84 %  SPIN 5 (  20 rpm)

85 > 88 %  SPIN 6 (  30 rpm)

89 > 92 %  SPIN 7 (  60 rpm)

93 > 96 %  SPIN 8 ( 100 rpm)

97 >100%  SPIN 9 ( 200 rpm)

OPTION : PERSONALITY DIP-SWITCH  N° 08 in position ON ( from 0 to 64% ) give access

to the PROPORTIONAL control of the color wheel with the following options:

-- Possibility of choice of any needed colour or any portion between one colour and

the adjacent one

-- Possibility to syncronize the colour changement with the other functions of the

projector  ( i.e. with the beam movement )

Note : Unchanged function from 65% to 100%

CHANNEL  2  =  1° GOBO WHEEL

percent values

1  >   4 %   GOBO 1 (OPEN)

5  >   8 %     ”   ”   ”   ”   ”   ”   + 3200° K

9  >  12 %    ”   ”   ”   ”   ”   ”   + 6000° K

13 >  16 %    ”   ”   ”   ”   ”   ”   + FROST

17 >  20 %    ”   ”   ”   ”   ”   ”   + PRISM

21 >  24 %   GOBO 2

25 >  28 %    ”   ”   ”   ”   ”   ”   + 3200° K

29 >  32 %    ”   ”   ”   ”   ”   ”   + 6000° K

33 >  36 %    ”   ”   ”   ”   ”   ”   + FROST

37 >  40 %    ”   ”   ”   ”   ”   ”   + PRISM

41 >  44 %   GOBO 3

45 >  48 %    ”   ”   ”   ”   ”   ”   + 3200° K

49 >  52 %    ”   ”   ”   ”   ”   ”   + 6000° K

53 >  56 %    ”   ”   ”   ”   ”   ”   + FROST

57 >  60 %    ”   ”   ”   ”   ”   ”   + PRISM

61 >  64 %   GOBO 4

65 >  68 %    ”   ”   ”   ”   ”   ”   + 3200° K

69 >  72 %    ”   ”   ”   ”   ”   ”   + 6000° K

73 >  76 %    ”   ”   ”   ”   ”   ”   + FROST

77 >  80 %    ”   ”   ”   ”   ”   ”   + PRISM

81 >  84 %  GOBO 5

85 >  88 %    ”   ”   ”   ”   ”   ”   + 3200° K

89 >  92 %    ”   ”   ”   ”   ”   ”   + 6000° K

93 >  96 %    ”   ”   ”   ”   ”   ”   + FROST

97 >100 %    ”   ”   ”   ”   ”   ”   + PRISM

CHANNEL  3  =  2° GOBO WHEEL

percent values

*   0   >   9   %   GOBO 1 (OPEN)

10 >  12  %   NEGATIVE ADJ. GOBO 2

*   13 >  16  %   GOBO 2

17 >  19  %   POSITIVE ADJ. GOBO 2

20 >  22  %   NEGATIVE ADJ. GOBO 3

*   23 >  26  %   GOBO 3

27 >  29  %   POSITIVE ADJ. GOBO 3

30 >  32  %   NEGATIVE ADJ. GOBO 4

*   33 >  36  %   GOBO 4

37 >  39  %   POSITIVE ADJ. GOBO 4

40 >  42  %   NEGATIVE ADJ. GOBO 5

*   43 >  46  %   GOBO 5

47 >  49  %   POSITIVE ADJ. GOBO 5

50 >  52  %   NEGATIVE ADJ. GOBO 6

*   53 >  56  %   GOBO 6

57 >  59  %   POSITIVE ADJ. GOBO 6

60 >  64  %   FORWARD SPIN 1  (    2 rpm )

65 >  67  %   FORWARD SPIN 2  (    4 rpm )

68 >  70  %   FORWARD SPIN 3  (   10 rpm )

71 >  73  %   FORWARD SPIN 4  (   35 rpm )

74 >  76  %   FORWARD SPIN 5  (   80 rpm )

77 >  79  %   FORWARD SPIN 6  (  150 rpm )

80 >  82  %   STOP

83 >  85  %   REVERSE SPIN 1    (    4 rpm )

86 >  88  %   REVERSE SPIN 2    (    8 rpm )

89 >  91  %   REVERSE SPIN 3    (   15 rpm )

92 >  94  %   REVERSE SPIN 4    (   30 rpm )

95 >  97  %   REVERSE SPIN 5    (   50 rpm )

98 > 100 %   REVERSE SPIN 6    (  100 rpm )

CHANNEL  4  =  GOBO  ( INDEX and ROTATION )

percent values

0 >  58,6 %   INDEXED POSITION  540°

control of the indexed position of the pattern on 540°

58,6 > 61   %   STOP

62 > 64   %   FORWARD SPIN 1    (    9 rpm )

65 > 67   %   FORWARD SPIN 2    (   15 rpm )

68 > 70   %   FORWARD SPIN 3    (   25 rpm )

71 > 73   %   FORWARD SPIN 4    (   50 rpm )

74 > 76   %   FORWARD SPIN 5    (   90 rpm )

77 > 79   %   FORWARD SPIN 6    (  120 rpm )

80 > 82   %   STOP

83 > 85   %   REVERSE SPIN 1     (    9 rpm )

86 > 88   %   REVERSE SPIN 2     (   15 rpm )

89 > 91   %   REVERSE SPIN 3     (   25 rpm )

92 > 94   %   REVERSE SPIN 4     (   50 rpm )

95 > 97   %   REVERSE SPIN 5     (   90 rpm )

98 >100  %   REVERSE SPIN 6     (  120 rpm )

CHANNEL 5  =  IRIS

percent values			                                                                                               0 %  >>>    CLOSED

>>>  100 %  OPEN

|OPTION  : PERSONALITY DIP-SWITCH

|	N° 05 in position ON

|    0 %  >>>    CLOSED

|   >>>  50 %   OPEN

|  (51 >  54 %  OPEN

55 >  62 %  PULSE EFFECT SPEED 1    (   10 rpm )

63 >  70 %    ”   ”   ”   ”   ”   ”   ”   ”   ”   2    (    20 rpm )

71 >  78 %    ”   ”   ”   ”   ”   ”   ”   ”   ”   3    (    30 rpm )

79 >  86 %    ”   ”   ”   ”   ”   ”   ”   ”   ”   4    (    45 rpm )

87 >  94 %    ”   ”   ”   ”   ”   ”   ”   ”   ”   5    (    60 rpm )

95 > 100%    ”   ”   ”   ”   ”   ”   ”   ”   ”   6    (    90 rpm )

CHANNEL 6= FOCUSING

percent values

0   %       CLOSE FOCUS

100 %      FOCUS TO INFINITE

CHANNEL  7  =  DIMMER / SHUTTER

percent values

0  %   BLACK-OUT

> <     INCREASING DIMMER LEVEL

45 >  50 %   DIMMER LEVEL 100 % ( OPEN )

51 >  55 %   STROBE EFFECT SPEED 1   ( 1 FLASH / SEC )

56 >  60 %    ”   ”   ”   ”   ”   ”   ”   ”   ”   ”   2   ( 2 FLASH / SEC )

61 >  65 %    ”   ”   ”   ”   ”   ”   ”   ”   ”   ”   3   ( 3 FLASH / SEC )

66 >  70 %    ”   ”   ”   ”   ”   ”   ”   ”   ”   ”   4   ( 4 FLASH / SEC )

71 >  75 %    ”   ”   ”   ”   ”   ”   ”   ”   ”   ”   5   ( 5 FLASH / SEC )

76 >  80 %    ”   ”   ”   ”   ”   ”   ”   ”   ”   ”   6   ( 6 FLASH / SEC )

81 >  85 %    ”   ”   ”   ”   ”   ”   ”   ”   ”   ”   7   ( 7 FLASH / SEC )

86 >  90 %    ”   ”   ”   ”   ”   ”   ”   ”   ”   ”   8   ( 8 FLASH / SEC )

91 >  95 %    ”   ”   ”   ”   ”   ”   ”   ”   ”   ”   9   ( 9 FLASH / SEC )

96 > 100%   DIMMER LEVEL 100% (OPEN)

CHANNEL  8  =  LAMP REMOTE CONTROL + SPECIAL FUNCTIONS

percent values

0  >  30  %   LAMP OFF

31 >  40  %   SELF-TEST AFTER 15 SEC.

41 >  44  %   No function

45 >  55  %   RESET  AFTER 5 SEC.

56 >  70  %   No function

71 > 100 %   LAMP ON

OPTION : PERSONALITY DIP-SWITCH # 10 IN POSITION ON ENABLE PERMANENT

LAMP ON  KEEPING UNCHANGED CONTROL ON THE OTHER FUNCTIONS.

CHANNEL  9  =  PAN

percent values

0 %   >>>    LEFT

>>>  100 %  RIGHT

CHANNEL  10  =  TILT

percent values

0 %   >>>    DOWN

>>>  100 %  UP