Infinity Club 1200    04 February 98

DMX PROTOCOL CHANNEL ASSIGNEMENT

| Channel | Function |  |  |
| --- | --- | --- | --- |
| 1 | COLOR |  |  |
| 2 | GOBO WHEEL |  |  |
| 3 | GOBO INDEX + ROTATION |  |  |
| 4 | EFFECTS WHEEL + IRIS |  |  |
| 5 | EFFECTS INDEX + ROTATION |  |  |
| 6 | DIMMER |  |  |
| 7 | SHUTTER |  |  |
| 8 | SPECIAL FUNCTIONS |  |  |
| 9 | PAN (Coarse) |  | PAN (Coarse) |
| 10 | 2° PAN (Fine) | Optional | TILT (Coarse) |
| 11 | TILT (Coarse) | functioning | 2° PAN (Fine) |
| 12 | 2° TILT (Fine) |  | 2° TILT (Fine) |

CHANNEL  1  =  COLOUR

percent values				decimal values

1  >  4 %   WHITE				 	=0

5  >  8 %   WHITE / RED				=16

9  > 12 %  RED					=25

13 > 16 %  RED / YELLOW				=35

17 > 20 %  YELLOW					=47

21 > 24 %  YELLOW / MAGENTA		 	=56

25 > 28 %  MAGENTA					=66

29 > 32 %  MAGENTA / GREEN			=77

33 > 36 %  GREEN					=87

37 > 40 %  GREEN / ORANGE				=96

41 > 44 %  ORANGE					=107

45 > 48 %  ORANGE / BLUE				=118

49 > 52 %  BLUE					=128

53 > 56 %  BLUE / WOOD         			=137

57 > 60 %  WOOD					=147

61 > 64 %  WOOD / WHITE				=158

65 % >>>   MAX FORWARDING SPIN			=166

>>> 80 %  MIN FORWARDING SPIN 			=204

81 > 83 %  STOP				 	=209

84 % >>>   MIN REVERSE SPIN		 	=215

>>>100%  MAX REVERSE SPIN			=255

OPTION : PERSONALITY DIP-SWITCH  N° ?? in position ON ( from 0 to 64% ) give access

to the PROPORTIONAL control of the color wheel

Note : Unchanged function from 65% to 100%

CHANNEL  2  =  GOBO WHEEL

percent values				decimal values

1  > 20 %   GOBO 1 (OPEN)				=0

21 > 40 %   GOBO 2					=80

41 > 60 %   GOBO 3					=128

61 > 80 %   GOBO 4					=180

81 >100 %  GOBO 5					=255

OPTION : PERSONALITY DIP-SWITCH N° ?? in position ON

Enable the gobo changer to work in black out while changing gobo (only the 1° gobo wheel)

CHANNEL 3  =  GOBO  ( INDEX and ROTATION )

percent values				decimal values

0 %  >>>    INDEXED POSITION 			=0

control of the indexed position of the pattern on 580° (1 and 1/2  revolution of the pattern)

>>>  60 %   INDEXED POSITION			=153

61 %  >>>    MAX FORWARD SPIN			=156

>>>  78 %   MIN FORWARD SPIN			=198

79 >  82 %   STOP					=204

83 %  >>>    MIN REVERSE  SPIN		 	=212

>>> 100 %  MAX REVERSE  SPIN		 	=255

CHANNEL  4 =  EFFECTS  WHEEL +IRIS

percent values				                                            decimal values

1  > 20 %   1 OPEN                             (IRIS decrease from OPEN to close )        =0

21 > 40 %   2 COL. TEMP. 3200°K       (IRIS decrease from OPEN to close )        =80

41 > 60 %   3 COL. TEMP. 6000°K       (IRIS decrease from OPEN to close )        =128

61 > 80 %   4 MULTIFACE PRISM       (IRIS decrease from OPEN to close )        =180

81 >100 %  5 FROST FILTER	           (IRIS decrease from OPEN to close )	   =255

CHANNEL  5  =  EFFECTS ( INDEX and ROTATION )

percent values				decimal values

0 %  >>>    INDEXED POSITION 			=0

control of the indexed position of the pattern on 580° (1 and 1/2  revolution of the pattern)

>>>  60 %   INDEXED POSITION			=153

61 %  >>>    MAX FORWARD SPIN			=156

>>>  78 %   MIN FORWARD SPIN			=198

79 >  82 %   STOP					=204

83 %  >>>    MIN REVERSE  SPIN		 	=212

>>> 100 %  MAX REVERSE  SPIN		 	=255

CHANNEL  6  =  DIMMER

percent values				decimal values

0  %   BLACK-OUT					=0

> <     INCREASING DIMMER LEVEL

100 %  DIMMER LEVEL 100% (OPEN)		=255

OPTION : PERSONALITY DIP-SWITCH

N° ?? in position ON

0  %   DIMMER LEVEL 100% (OPEN)		=0

> <     DECREASING DIMMER LEVEL

100 %  BLACK-OUT					=255

CHANNEL  7  =  SHUTTER

percent values				decimal values

0  > 10 %  CLOSED					=0

11 > 20 %  OPEN					=40

21 %  >>>  STROBE EFFECT MIN SPEED		=54

>>> 100 % STROBE EFFECT MAX SPEED		=255

OPTION : PERSONALITY DIP-SWITCH

N° ?? in position ON

0  > 10 %  OPEN					=0

11 > 20 %  CLOSED					=40

21 %  >>>  STROBE EFFECT MIN SPEED		=54

>>> 100 % STROBE EFFECT MAX SPEED		=255

CHANNEL  8  =  SPECIAL FUNCTIONS

percent values				decimal values

0  >  30  %   LAMP OFF AFTER 5 SEC.		=0

31 >  40  %   SELF-TEST AFTER 15 SEC.       	=90

41 >  44  %   No function

45 >  55  %   RESET  AFTER 5 SEC.			=128

56 >  70  %   No function

71 > 100 %   LAMP ON				=255

OPTION : PERSONALITY DIP-SWITCH # ?? IN POSITION ON ENABLE PERMANENT

LAMP ON  KEEPING UNCHANGED CONTROL ON THE OTHER FUNCTIONS.

# NOTE: LAMP ON and LAMP OFF WILL BE ENABLED AGAIN WHEN POSSIBLE

CHANNEL  9  =  PAN

percent values				decimal values

0 %   >>>    LEFT					=0

>>>  100 %  RIGHT					=255

OPTION  : PERSONALITY DIP-SWITCH

N° ?? in position ON

0 %   >>>    RIGHT					=0

>>>  100 %  LEFT					=255

CHANNEL  10  =  TILT

percent values				decimal values

0 %   >>>     UP					=0

>>>  100 %   DOWN					=255

OPTION  : PERSONALITY DIP-SWITCH

N° ?? in position ON

0 %   >>>    DOWN					=0

>>>  100 %  UP					=255

# NOTE : WITH PERSONALITY DIP-SWITCH N° 01 IN POSITION ON

OPTIONAL CHANNEL 11= 2° PAN CHANNEL

OPTIONAL CHANNEL 12= 2° TILT CHANNEL

# NOTE : WITH PERSONALITY DIP-SWITCH N° 01,02,03 IN POSITION ON

CHANNEL    9= PAN (Coarse)

CHANNEL  10= 2° PAN (Fine)

CHANNEL  11= TILT (Coarse)

CHANNEL  12= 2° TILT (Fine)           Prefer this option with 16-bit-movement