Infinity Live 1200    21 JANUARY 97

DMX PROTOCOL CHANNEL ASSIGNEMENT

| Channel | Function | Channel | Function |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | DIMMER | 11 | EFFECTS WHEEL |  |  |
| 2 | SHUTTER | 12 | COLOR WHEEL |  |  |
| 3 | IRIS | 13 | CYAN |  |  |
| 4 | ZOOM | 14 | YELLOW |  |  |
| 5 | FOCUS | 15 | MAGENTA |  |  |
| 6 | GOBO WHEEL (Static) | 16 | LAMP ON/OFF + SPECIAL FUNCTIONS |  |  |
| 7 | GOBO WHEEL (Rotate) | 17 | PAN (Coarse) |  | PAN (Coarse) |
| 8 | GOBO INDEX + ROTATION | 18 | 2° PAN (Fine) | Optional | TILT (Coarse) |
| 9 | PRISM WHEEL (Rotate) | 19 | TILT (Coarse) | functioning | 2° PAN (Fine) |
| 10 | PRISM INDEX + ROTATION | 20 | 2° TILT (Fine) |  | 2° TILT (Fine) |

CHANNEL 1  =  DIMMER

percent values				decimal values

0  %   BLACK-OUT					=0

> <     INCREASING DIMMER LEVEL

100 %  DIMMER LEVEL 100% (OPEN)		=255

|OPTION : WITH ACTIVE OPTION N° 07

|    0  %   DIMMER LEVEL 100% (OPEN)		=0

|    > <     DECREASING DIMMER LEVEL

|  100 %  BLACK-OUT					=255

CHANNEL  2  =  SHUTTER

percent values				decimal values

0  > 10 %  CLOSED					=0

11 > 20 %  OPEN					=40

21 %  >>>  STROBE EFFECT MIN SPEED		=54

>>> 100 % STROBE EFFECT MAX SPEED		=255

|OPTION : WITH ACTIVE OPTION N° 06

|   0  > 10 %  OPEN					=0

|  11 > 20 %  CLOSED					=40

|  21 %  >>>  STROBE EFFECT MIN SPEED		=54

|  >>> 100 % STROBE EFFECT MAX SPEED		=255

CHANNEL  3  =  IRIS

percent values				decimal values

0 %  >>>    CLOSED					=0

>>>  100 %  OPEN					=255

|OPTION  : WITH ACTIVE OPTION N° 08

|    0 %  >>>    OPEN					=0

|   >>>  50 %   CLOSED					=128

|  (51 >  54 %  CLOSED)				=133

|    55 % >>>   PULSE EFFECT minimum  Speed	=141

|   >>>   94 %  PULSE EFFECT maximum Speed	=239

|   95 > 100%  CLOSED

CHANNEL  4  =  ZOOM

percent values				decimal values

0 %  >>>   NARROW BEAM				=0

>>> 100%  WIDE BEAM				=255

CHANNEL  5  =  FOCUS

percent values				decimal values

0 %  >>>   CLOSE FOCUS				=0

>>> 100%  FOCUS TO INFINITE			=255

CHANNEL  6  =  GOBO WHEEL (STATIC)

percent values				decimal values

0   >   9   %   GOBO 1 (OPEN)			=0

13 >  16  %   GOBO 2					=37

23 >  26  %   GOBO 3  Dichro-Sector			=64

33 >  36  %   GOBO 4					=88

43 >  46  %   GOBO 5  TUNNEL			=112

53 >  56  %   GOBO 6					=136

60 %  >>>     MAX FORWARD SPIN	 		=153

>>>  79  %   MIN FORWARD SPIN 	 		=201

80 >  81  %   STOP					=206

82 %  >>>     MIN REVERSE  SPIN 		 	=210

>>>  100 %   MAX REVERSE  SPIN 		 	=255

OPTION : WITH ACTIVE OPTION  N° 11 PROPORTIONAL CONTROL OF THE STATIC GOBO WHEEL

Note : Unchanged function from 60% to 100%

CHANNEL  7  =  GOBO WHEEL (ROTATE)

percent values				decimal values

1  > 20 %   GOBO 1 (OPEN)				=0

21 > 40 %   GOBO 2					=80

41 > 60 %   GOBO 3					=128

61 > 80 %   GOBO 4					=180

81 >100 %  GOBO 5					=255

OPTION : WITH ACTIVE OPTION  N° 10 THE GOBO CHANGE IN BLACK OUT

(ONLY THE ROTATING GOBO WHEEL)

CHANNEL  8  =  GOBO INDEX + ROTATION

percent values				decimal values

0 %  >>>    INDEXED POSITION 			=0

control of the indexed position of the pattern on 580° (1 and 1/2  revolution of the pattern)

>>>  60 %   INDEXED POSITION			=153

61 %  >>>    MAX FORWARD SPIN			=156

>>>  78 %   MIN FORWARD SPIN			=198

79 >  82 %   STOP					=204

83 %  >>>    MIN REVERSE  SPIN		 	=212

>>> 100 %  MAX REVERSE  SPIN		 	=255

CHANNEL  9  =  PRISM  WHEEL (ROTATE)

percent values				decimal values

1  > 25 %   OPEN  					=0

26 > 50 %   PRISM 1					=100

51 > 75 %   PRISM 2					=160

76 >100 %  PRISM 3					=255

OPTION : WITH ACTIVE OPTION  N° 09 THE PRISM CHANGE IN BLACK OUT

(ONLY THE ROTATING GOBO WHEEL)

CHANNEL  10  =  PRISM INDEX + ROTATION

percent values				decimal values

0 %  >>>    INDEXED POSITION 			=0

control of the indexed position of the pattern on 580° (1 and 1/2  revolution of the pattern)

>>>  60 %   INDEXED POSITION			=153

61 %  >>>    MAX FORWARD SPIN			=156

>>>  78 %   MIN FORWARD SPIN			=198

79 >  82 %   STOP					=204

83 %  >>>    MIN REVERSE  SPIN		 	=212

>>> 100 %  MAX REVERSE  SPIN		 	=255

CHANNEL  11  =  EFFECTS WHEEL

percent values				decimal values

1  > 20 %   1 OPEN 					=0

21 > 40 %   2 FROST  1				=80

41 > 60 %   3 FROST  2	 			=128

61 > 80 %   4 EMPTY					=180

81 >100 %  5 MULTIFACE PRISM			=255

CHANNEL  12  =  COLOUR  WHEEL

percent values				decimal values

1  >  4 %   WHITE				 	=0

5  >  8 %   WHITE / RED				=16

9  > 12 %  RED					=25

13 > 16 %  RED / YELLOW				=35

17 > 20 %  YELLOW					=47

21 > 24 %  YELLOW / MAGENTA		 	=56

25 > 28 %  MAGENTA					=66

29 > 32 %  MAGENTA / GREEN			=77

33 > 36 %  GREEN					=87

37 > 40 %  GREEN / ORANGE				=96

41 > 44 %  ORANGE					=107

45 > 48 %  ORANGE / BLUE				=118

49 > 52 %  BLUE					=128

53 > 56 %  BLUE / PINK				=137

57 > 60 %  PINK					=147

61 > 64 %  PINK / WHITE				=158

65 % >>>   MAX FORWARDING SPIN			=166

>>> 80 %  MIN FORWARDING SPIN 			=204

81 > 83 %  STOP				 	=209

84 % >>>   MIN REVERSE SPIN		 	=215

>>>100%  MAX REVERSE SPIN			=255

OPTION : WITH ACTIVE OPTIO N° 12 PROPORTIONAL CONTROL OF THE COLOR WHEEL

Note : Unchanged function from 65% to 100%

CHANNEL  13  =  CYAN

percent values				decimal values

0 %  >>>   OPEN					=0

>>> 100%  CYAN					=255

CHANNEL  14  =  YELLOW

percent values				decimal values

0 %  >>>   OPEN					=0

>>> 100%  YELLOW					=255

CHANNEL  15  =  MAGENTA

percent values				decimal values

0 %  >>>   OPEN					=0

>>> 100%  MAGENTA					=255

OPTION TO CHANNEL  13,  14  AND  15 :  WITH ACTIVE OPTION N° 16  INVERT THE CHANNELS

FUNCTIONING GIVING SATURATED COLORS AT 0% AND OPEN AT 100%

CHANNEL  16  =  LAMP REMOTE CONTROL + SPECIAL FUNCTIONS

percent values				decimal values

0  >  30  %   LAMP OFF AFTER 5 SEC.		=0

31 >  40  %   SELF-TEST AFTER 15 SEC.       	=90

41 >  44  %   No function

45 >  55  %   RESET  AFTER 5 SEC.			=128

56 >  70  %   No function

71 > 100 %   LAMP ON				=255

OPTION : WITH ACTIVE OPTION N° 01 PERMANENT LAMP ON  KEEPING UNCHANGED CONTROL ON

THE OTHER FUNCTIONS.

CHANNEL  17  =  PAN

percent values				decimal values

0 %   >>>    LEFT					=0

>>>  100 %  RIGHT					=255

| OPTION  : WITH ACTIVE OPTION N° 05

|   0 %   >>>    RIGHT					=0

|  >>>  100 %  LEFT					=255

| (16 bit HIGH DEFINITION DMX with active option n° 19 or 20 )

CHANNEL  18  =  TILT

percent values				decimal values

0 %   >>>     UP					=0

>>>  100 %   DOWN					=255

| OPTION  : WITH ACTIVE OPTION N° 04

|   0 %   >>>    DOWN					=0

|  >>>  100 %  UP					=255

|(16 bit HIGH DEFINITION DMX with active option n° 19 or 20 )

NOTE 1 : WITH ACTIVE OPTION N° 19

CHANNEL 17  =  PAN  (Coarse)

CHANNEL 18  =  TILT  (Coarse)

CHANNEL 19  = 2° PAN CHANNEL  ( Fine)

CHANNEL 20  = 2° TILT CHANNEL  (Fine)

NOTE 2 : WITH ACTIVE OPTION N° 20

CHANNEL 17  =  PAN  (Coarse)

CHANNEL 18  =  2° PAN CHANNEL  (Fine)

CHANNEL 19  =  TILT  (Coarse)

CHANNEL 20  =  2° TILT CHANNEL  (Fine)

OPTION  01  	LAMP ON

OPTION  02  	SELF TEST

OPTION  03  	INVERT PAN WITH TILT

OPTION  04  	INVERT TILT

OPTION  05  	INVERT PAN

OPTION  06  	INVERT SHUTTER

OPTION  07  	INVERT DIMMER

OPTION  08	INVERT IRIS + IRIS PULSE

OPTION  09  	PRISM CHANGE IN BLACK OUT

OPTION  10  	ROTATING GOBO CHANGE IN BLACK OUT

OPTION  11  	STATIC GOBO WHEEL LINEAR CONTROL

OPTION  12 	COLOR WHEEL LINEAR CONTROL

OPTION  16 	INVERT RGB

OPTION  19	HIGH DEFINITION SCANNING OPTION 1

OPTION  20	HIGH DEFINITION SCANNING OPTION 2

DA DISPLAY E TASTIERA

INDIRIZZO DMX

DIAGNOSTICO DMX

ACCESSO ALLE OPZIONI

EFFICIENZA LAMPADA % CON RESET A 100% QUANDO SI MONTA LA LAMPADA DOPO AVER MONTATO UNA FOTORESISTENZA IN POSIZIONE STRATEGICA

INTERVENTO TERMOSTATO

FUNZIONE STAND BY

CONTAORE GENERALE

CONTAORE LAMPADA AZZERABILE