Infinity Multi Step Zoom 1200    14 JANUARY 97

DMX PROTOCOL CHANNEL ASSIGNEMENT

| Channel | Function |  |  |
| --- | --- | --- | --- |
| 1 | COLOR |  |  |
| 2 | 1° GOBO WHEEL |  |  |
| 3 | 2° GOBO WHEEL |  |  |
| 4 | GOBO INDEX + ROTATION |  |  |
| 5 | EFFECTS WHEEL |  |  |
| 6 | IRIS |  |  |
| 7 | MULTI STEP ZOOM + FOCUS |  |  |
| 8 | DIMMER |  |  |
| 9 | SHUTTER |  |  |
| 10 | LAMP ON/OFF + SPECIAL FUNCTIONS |  |  |
| 11 | PAN (Coarse) |  | PAN (Coarse) |
| 12 | 2° PAN (Fine) | Optional | TILT (Coarse) |
| 13 | TILT (Coarse) | functioning | 2° PAN (Fine) |
| 14 | 2° TILT (Fine) |  | 2° TILT (Fine) |

CHANNEL  1  =  COLOUR

percent values				decimal values

1  >  4 %   WHITE				 	=0

5  >  8 %   WHITE / RED				=16

9  > 12 %  RED					=25

13 > 16 %  RED / YELLOW				=35

17 > 20 %  YELLOW					=47

21 > 24 %  YELLOW / MAGENTA		 	=56

25 > 28 %  MAGENTA					=66

29 > 32 %  MAGENTA / GREEN			=77

33 > 36 %  GREEN					=87

37 > 40 %  GREEN / ORANGE				=96

41 > 44 %  ORANGE					=107

45 > 48 %  ORANGE / BLUE				=118

49 > 52 %  BLUE					=128

53 > 56 %  BLUE / PINK				=137

57 > 60 %  PINK					=147

61 > 64 %  PINK / WHITE				=158

65 % >>>   MAX FORWARDING SPIN			=166

>>> 80 %  MIN FORWARDING SPIN 			=204

81 > 83 %  STOP				 	=209

84 % >>>   MIN REVERSE SPIN		 	=215

>>>100%  MAX REVERSE SPIN			=255

OPTION : PERSONALITY DIP-SWITCH  N° 08 in position ON ( from 0 to 64% ) give access

to the PROPORTIONAL control of the color wheel

Note : Unchanged function from 65% to 100%

CHANNEL  2  =  1° GOBO WHEEL

percent values				decimal values

1  > 20 %   GOBO 1 (OPEN)				=0

21 > 40 %   GOBO 2					=80

41 > 60 %   GOBO 3					=128

61 > 80 %   GOBO 4					=180

81 >100 %  GOBO 5					=255

OPTION : PERSONALITY DIP-SWITCH N° 07 in position ON

Enable the gobo changer to work in black out while changing gobo (only the 1° gobo wheel)

CHANNEL  3  =  2° GOBO WHEEL

percent values				decimal values

*   0   >   9   %   GOBO 1 (OPEN)			=0

10 >  12  %   NEGATIVE ADJ. GOBO 2

*   13 >  16  %   GOBO 2				=40

17 >  19  %   POSITIVE ADJ. GOBO 2

20 >  22  %   NEGATIVE ADJ. GOBO 3

*   23 >  26  %   GOBO 3  Dichro-Sector			=64

27 >  29  %   POSITIVE ADJ. GOBO 3

30 >  32  %   NEGATIVE ADJ. GOBO 4

*   33 >  36  %   GOBO 4				=88

37 >  39  %   POSITIVE ADJ. GOBO 4

40 >  42  %   NEGATIVE ADJ. GOBO 5

*   43 >  46  %   GOBO 5  TUNNEL			=112

47 >  49  %   POSITIVE ADJ. GOBO 5

50 >  52  %   NEGATIVE ADJ. GOBO 6

*   53 >  56  %   GOBO 6				=136

57 >  59  %   POSITIVE ADJ. GOBO 6

60 %  >>>     MAX FORWARD SPIN	 		=153

>>>  79  %   MIN FORWARD SPIN 	 		=201

80 >  81  %   STOP					=206

82 %  >>>     MIN REVERSE  SPIN 		 	=210

>>>  100 %   MAX REVERSE  SPIN 		 	=255

CHANNEL  4  =  GOBO  ( INDEX and ROTATION )

percent values				decimal values

0 %  >>>    INDEXED POSITION 			=0

control of the indexed position of the pattern on 580° (1 and 1/2  revolution of the pattern)

>>>  60 %   INDEXED POSITION			=153

61 %  >>>    MAX FORWARD SPIN			=156

>>>  78 %   MIN FORWARD SPIN			=198

79 >  82 %   STOP					=204

83 %  >>>    MIN REVERSE  SPIN		 	=212

>>> 100 %  MAX REVERSE  SPIN		 	=255

CHANNEL  5  =  EFFECTS 	WHEEL

percent values				decimal values

1  > 20 %   1 OPEN 					=0

21 > 40 %   2 COL. TEMP. 3200°K			=80

41 > 60 %   3 COL. TEMP. 6000°K 			=128

61 > 80 %   4 MULTIFACE PRISM			=180

81 >100 %  5 FROST FILTER				=255

CHANNEL  6  =  IRIS

percent values				decimal values

0 %  >>>    CLOSED					=0

>>>  100 %  OPEN					=255

|OPTION  : PERSONALITY DIP-SWITCH

|	N° 06 in position ON

|    0 %  >>>    OPEN					=0

|   >>>  50 %   CLOSED					=128

|  (51 >  54 %  CLOSED)				=133

|    55 % >>>   PULSE EFFECT minimum  Speed	=141

|   >>>   94 %  PULSE EFFECT maximum Speed	=239

|   95 > 100%  CLOSED					=255

CHANNEL  7  =  MULTI STEP ZOOM + FOCUSING

percent values				decimal values

0 %  >>>  CLOSE FOCUS		|   Obj.	|	=0

>>> 25 %   FOCUS TO INFINITE	|   9.5°	|	=63

26 %  >>>  CLOSE FOCUS		|   Obj.	|	=64

>>> 50 %   FOCUS TO INFINITE	|   9.5°	|	=128

51 %  >>>  CLOSE FOCUS		|   Obj.	|	=129

>>> 75 %   FOCUS TO INFINITE	|   9.5°	|	=191

76 %  >>>  CLOSE FOCUS		|   Obj.	|	=192

>>> 100%  FOCUS TO INFINITE	|   9.5°	|	=255

CHANNEL  8  =  DIMMER

percent values				decimal values

0  %   BLACK-OUT					=0

> <     INCREASING DIMMER LEVEL

100 %  DIMMER LEVEL 100% (OPEN)		=255

OPTION : PERSONALITY DIP-SWITCH

N° 05  in position ON

0  %   DIMMER LEVEL 100% (OPEN)		=0

> <     DECREASING DIMMER LEVEL

100 %  BLACK-OUT					=255

CHANNEL  9  =  SHUTTER

percent values				decimal values

0  > 10 %  CLOSED					=0

11 > 20 %  OPEN					=40

21 %  >>>  STROBE EFFECT MIN SPEED		=54

>>> 100 % STROBE EFFECT MAX SPEED		=255

OPTION : PERSONALITY DIP-SWITCH

N° 04 in position ON

0  > 10 %  OPEN					=0

11 > 20 %  CLOSED					=40

21 %  >>>  STROBE EFFECT MIN SPEED		=54

>>> 100 % STROBE EFFECT MAX SPEED		=255

CHANNEL  10  =  LAMP REMOTE CONTROL + SPECIAL FUNCTIONS

percent values				decimal values

0  >  30  %   LAMP OFF AFTER 5 SEC.		=0

31 >  40  %   SELF-TEST AFTER 15 SEC.       	=90

41 >  44  %   No function

45 >  55  %   RESET  AFTER 5 SEC.			=128

56 >  70  %   No function

71 > 100 %   LAMP ON				=255

OPTION : PERSONALITY DIP-SWITCH # 10 IN POSITION ON ENABLE PERMANENT

LAMP ON  KEEPING UNCHANGED CONTROL ON THE OTHER FUNCTIONS.

CHANNEL  11  =  PAN

percent values				decimal values

0 %   >>>    LEFT					=0

>>>  100 %  RIGHT					=255

OPTION  : PERSONALITY DIP-SWITCH

N° 03 in position ON

0 %   >>>    RIGHT					=0

>>>  100 %  LEFT					=255

CHANNEL  12  =  TILT

percent values				decimal values

0 %   >>>     UP					=0

>>>  100 %   DOWN					=255

OPTION  : PERSONALITY DIP-SWITCH

N° 02 in position ON

0 %   >>>    DOWN					=0

>>>  100 %  UP					=255

# NOTE : WITH PERSONALITY DIP-SWITCH N° 01 IN POSITION ON

OPTIONAL CHANNEL 13= 2° PAN CHANNEL

OPTIONAL CHANNEL 14= 2° TILT CHANNEL

# NOTE : WITH PERSONALITY DIP-SWITCH N° 01,02,03 IN POSITION ON

CHANNEL  11= PAN (Coarse)

CHANNEL  12= 2° PAN (Fine)

CHANNEL  13= TILT (Coarse)

CHANNEL  14= 2° TILT (Fine)           Prefer this option with 16-bit-movement

OPTION 10 LAMP ON

OPTION  9  SELF TEST

OPTION  8  COLOR WHEEL LINEAR CONTROL

OPTION  7  GOBO CHANGE IN BLACK OUT

OPTION  6  INVERT IRIS + IRIS PULSE

OPTION  5  INVERT DIMMER

OPTION  4  INVERT SHUTTER

OPTION  3  INVERT PAN

OPTION  2  INVERT TILT

OPTION  1  HIGH DEFINITION SCANNING