SUPER PRINCE 575  TEMPLATE          14/01/97

10 CHANNELS DMX PROTOCOL

| Channel | Function |
| --- | --- |
| 1 | COLOR |
| 2 | GOBO WHEEL |
| 3 | GOBO INDEX + ROTATION |
| 4 | EFFECTS WHEEL |
| 5 | IRIS |
| 6 | DIMMER |
| 7 | SHUTTER |
| 8 | SPECIAL FUNCTIONS |
| 9 | PAN |
| 10 | TILT |

NOTE: HIGH DEFINITION SCANNING  (OPTIONAL)

# Ch         Function

9           PAN (Coarse)

- PAN (Fine)

- TILT (Coarse)

- TILT (Fine)

To enable this option set in position ON the dip-switch #10

of the DMX SECTION (Unproperly named ” 6 channel dmx protocol ”)

CHANNEL  1  =  COLOUR

percent values				decimal values

1  >  4 %   WHITE					=0

5  >  8 %   WHITE / RED				=16

9  > 12 %  RED					=26

13 > 16 %  RED / YELLOW				=36

17 > 20 %  YELLOW					=46

21 > 24 %  YELLOW / CYAN				=56

25 > 28 %  CYAN					=67

29 > 32 %  CYAN / MAGENTA				=77

33 > 36 %  MAGENTA					=87

37 > 40 %  MAGENTA / GREEN			=98

41 > 44 %  GREEN					=108

45 > 48 %  GREEN / BLUE				=118

49 > 52 %  BLUE				 	=128

53 > 56 %  BLUE / WHITE			 	=139

57 > 60 %  NO FUNCTION				=149

61 % >>>   MINIMUM SPIN				=159

>>> 100%  MAXIMUM SPIN				=255

OPTION : PERSONALITY DIP-SWITCH  N° 08 in position ON ( from 0 to 60% ) give access

to the PROPORTIONAL control of the color wheel

Note : Unchanged function from 61% to 100%

CHANNEL  2  =  GOBO WHEEL

percent values				decimal values

1  > 20 %   GOBO 1 (OPEN)				=0

21 > 40 %   GOBO 2					=80

41 > 60 %   GOBO 3					=128

61 > 80 %   GOBO 4					=180

81 >100 %  GOBO 5					=255

OPTION : PERSONALITY DIP-SWITCH

N° 07 in position ON

Enable the gobo changer to work in black out while changing gobo

CHANNEL  3  =  GOBO ROTATION

percent values				decimal values

0%  >  >  >       INDEXED POSITION 			=0

control of the indexed position of the pattern on 540° (1 and 1/2  revolution of the pattern)

>   >   >  60%   INDEXED POSITION			=150

61 %  >>>    MAX FORWARD SPIN		 	=154

>>>  76  %   MIN FORWARD SPIN			=190

77 >  83  %   STOP					=202

84 %  >>>    MIN REVERSE  SPIN			=216

>>> 100 %   MAX REVERSE  SPIN			=255

CHANNEL  4  =  EFFECT WHEEL

percent values				decimal values

1  >  16   % 	1 OPEN				=0

17 >  34  % 	2 COL. TEMP. 3200°K			=67

35 >  50  % 	3 COL. TEMP. 6000°K			=95

51 >  66  %  	4 COL. CORR. PINK			=150

67 >  84  %  	5 FROST FILTER			=193

85 > 100 %  	6 MULTIFACE PRISM			=255

CHANNEL  5  =  IRIS

percent values				decimal values

0 % >>>   CLOSED					=0

>>> 100 % OPEN					=255

OPTION  : PERSONALITY DIP-SWITCH

N° 06 in position ON

0 %  >>>   OPEN					=0

>>>  50 %  CLOSED					=128

(51 >  54%  CLOSED)				=133

55 % >>>   PULSE EFFECT minimum  Speed	=141

>>>  96 %  PULSE EFFECT maximum Speed	=244

97 > 100% CLOSED					=255

CHANNEL  6  =  DIMMER

percent values				decimal values

0  %   BLACK-OUT					=0

> <     INCREASING DIMMER LEVEL

100 %  DIMMER LEVEL 100% (OPEN)		=255

OPTION : PERSONALITY DIP-SWITCH

N° 05  in position ON

0  %   DIMMER LEVEL 100% (OPEN)		=0

> <     DECREASING DIMMER LEVEL

100 %  BLACK-OUT					=255

CHANNEL  7  =  SHUTTER

percent values				decimal values

0  > 10 %  OPEN					=0

11 > 20 %  CLOSED					=40

21 %  >>>  STROBE EFFECT MIN SPEED		=54

>>> 100 % STROBE EFFECT MAX SPEED		=255

OPTION : PERSONALITY DIP-SWITCH

N° 04 in position ON

0  > 10 %  CLOSED					=0

11 > 20 %  OPEN					=40

21 %  >>>  STROBE EFFECT MIN SPEED		=54

>>> 100 % STROBE EFFECT MAX SPEED		=255

CHANNEL  8  =  LAMP REMOTE CONTROL + SPECIAL FUNCTIONS

percent values				decimal values

0  >  30  %   LAMP OFF AFTER 20 SEC.		=0

31 >  40  %   SELF-TEST AFTER 20 SEC.        	=90

41 >  44  %   No function

45 >  55  %   RESET  AFTER 5 SEC.			=128

56 >  70  %   No function

71 > 100 %   LAMP ON				=255

OPTION : PERSONALITY DIP-SWITCH

N° 10 IN POSITION ON

ENABLE PERMANENT LAMP ON  KEEPING UNCHANGED CONTROL ON THE OTHER FUNCTIONS

CHANNEL  9  =  PAN

percent values				decimal values

0 %   >>>    LEFT					=0

>>>  100 %  RIGHT					=255

OPTION  : PERSONALITY DIP-SWITCH

N° 03 in position ON

0 %   >>>    RIGHT					=0

>>>  100 %  LEFT					=255

CHANNEL  10  =  TILT

percent values				decimal values

0 %   >>>     UP					=0

>>>  100 %   DOWN					=255

OPTION  : PERSONALITY DIP-SWITCH

N° 02 in position ON

0 %   >>>    DOWN					=0

>>>  100 %  UP					=255

OPTION : PERSONALITY DIP-SWITCH

N° 01  in position ON invert the channel functioning of the PAN with the TILT keeping unchanged 	functions and eventual options utilized with dip-switches 02 & 03.