MASK COLOR ZOOM   TEMPLATE          03/01/97

PROCEDURA DI SELF-TEST DEL MASK COLOR ZOOM 1200

COMANDO DA DMX

Dopo il reset apre dimmer e iride

--Dimmer lentamente da 100% a 0% a 100%   x2

Sequenza strobo a velocità crescente e decrescente   x2

Iride chiude e apre con sequenze velocità fino alla massima   x2

Zoom sequenza da piccolo a grande   x2

Focus quando lo zoom è piccolo fa una corsa completa avanti e indietro  x2

Focus uguale a zoom grande  x2

Colore Magenta da bianco a saturo lentamente e velocemente   x2

Uguale per Blue e Giallo

Rimane sul giallo e zoomma come prima   x2

A zoom piccolo entra lentamente il frost e quando il frost è tutto chiuso zoomma lentamente a grande  x2

--Riparte da dimmer senza resettare

Quando viene disabilitato il self-test fa il reset completo

CON COMANDO DA DIP-SWITCH SUL PANNELLO POSTERIORE

CON LAMP-ON SU ON UGUALE A SOPRA

CON LAMP ON SU OFF PRIMA ACCENDERE LA LAMPADA CHE VERRà SPENTA DOPO 20 SECONDI DALLA DISATTIVAZIONE DEL DIP-SWITCH DEL SELF-TEST.

| Channel | Function |
| --- | --- |
| 1 | CYAN |
| 2 | YELLOW |
| 3 | MAGENTA |
| 4 | FROST |
| 5 | IRIS |
| 6 | ZOOM |
| 7 | FOCUS |
| 8 | SPECIAL FUNCTIONS |
| 9 | SHUTTER |
| 10 | DIMMER |

CHANNEL  1  =  CYAN

percent values				decimal values

0 %  >>>   OPEN					=0

>>> 100%  CYAN					=255

CHANNEL  2  =  YELLOW

percent values				decimal values

0 %  >>>   OPEN					=0

>>> 100%  YELLOW					=255

CHANNEL  3  =  MAGENTA

percent values				decimal values

0 %  >>>   OPEN					=0

>>> 100%  MAGENTA					=255

OPTION TO CHANNEL  1,  2  AND  3 :  PERSONALITY DIP-SWITCH

N° 09 in position ON

Invert the channels functioning giving saturated colors at 0% and Open at 100%

CHANNEL  4  =  FROST

percent values				decimal values

0 %  >>>   OPEN					=0

>>> 100%  FROST					=255

OPTION :  PERSONALITY DIP-SWITCH

N° 08 in position ON

0 %  >>>   FROST					=0

>>> 100%  OPEN					=255

CHANNEL  5  IRIS

percent values				decimal values

0   %  CLOSED					=0

> <

100  %  OPEN						=255

|OPTION 1 : PERSONALITY DIP-SWITCH

|	N° 07 in position ON

| percent values					decimal values

|    0   %  OPEN						=0

|       > <

|  100  %  CLOSED					=255

|OPTION 2 : PERSONALITY DIP-SWITCH

|	N° 06 in position ON

|     0   %  CLOSED					=0

|      > <

|     50   %  OPEN					=128

|    (51 >  54 %  OPEN)					=133

|     55 % >>>   PULSE EFFECT minimum  Speed	=141

|     >>>   94 %	PULSE EFFECT maximum Speed	=239

|     95 > 100%  OPEN					=255

|OPTION 3 : PERSONALITY DIP-SWITCHES

|	N° 06 & 07  in position ON

|     0   %    OPEN					=0

|      > <

|     50   %  CLOSED					=128

|    (51 >  54 %  CLOSED)				=133

|     55 % >>>   PULSE EFFECT minimum  Speed	=141

|     >>>   94 %	PULSE EFFECT maximum Speed	=239

|     95 > 100%  CLOSED				=255

CHANNEL  6  =  ZOOM

percent values				decimal values

0 %  >>>   NARROW BEAM				=0

>>> 100%  WIDE BEAM				=255

CHANNEL  7  =  FOCUS

percent values				decimal values

0 %  >>>   CLOSE FOCUS				=0

>>> 100%  FOCUS TO INFINITE			=255

CHANNEL  8  =  LAMP REMOTE CONTROL + SPECIAL FUNCTIONS

percent values				decimal values

0  >  30  %   LAMP OFF AFTER 20 SEC.		=0

31 >  40  %   SELF-TEST AFTER 20 SEC.        	=90

41 >  44  %   No function

45 >  55  %   RESET  AFTER 5 SEC.			=128

56 >  70  %   No function

71 > 100 %   LAMP ON				=255

OPTION : PERSONALITY DIP-SWITCH

N° 10 IN POSITION ON

ENABLE PERMANENT LAMP ON  KEEPING UNCHANGED CONTROL ON THE OTHER FUNCTIONS

CHANNEL  9  =  SHUTTER

percent values				decimal values

0  > 10 %  OPEN					=0

11 > 20 %  CLOSED					=40

21 %  >>>  STROBE EFFECT MIN SPEED		=54

>>> 100 % STROBE EFFECT MAX SPEED		=255

OPTION : PERSONALITY DIP-SWITCH

N° 04 in position ON

0  > 10 %  CLOSED					=0

11 > 20 %  OPEN					=40

21 %  >>>  STROBE EFFECT MIN SPEED		=54

>>> 100 % STROBE EFFECT MAX SPEED		=255

CHANNEL  10  =  DIMMER

percent values				decimal values

0  %   BLACK-OUT					=0

> <     INCREASING DIMMER LEVEL

100 %  DIMMER LEVEL 100% (OPEN)		=255

OPTION : PERSONALITY DIP-SWITCH

N° 05  in position ON

0  %   DIMMER LEVEL 100% (OPEN)		=0

> <     DECREASING DIMMER LEVEL

100 %  BLACK-OUT					=255

OPTION 10 LAMP ON

OPTION  9  SELF-TEST

OPTION  8  INVERT COLOR

OPTION  7  INVERT IRIS

OPTION  6  IRIS PULSE

OPTION  5  INVERT DIMMER

OPTION  4  INVERT SHUTTER