TRACER

25 SEPT 95

DMX PROTOCOL CHANNEL ASSIGNEMENT

CHANNEL  1  =  COLOUR

percent values				decimal values

1  >  4 %   WHITE				F(White) 	=0

5  >  8 %   WHITE / RED			F(Whi/Red)	=16

9  > 12 %  RED				F(Red) 		=25

13 > 16 %  RED / YELLOW			F(Red/Yell) 	=35

17 > 20 %  YELLOW				F(Yellow) 	=47

21 > 24 %  YELLOW / MAGENTA		F(Yel/Pur) 	=56

25 > 28 %  MAGENTA				F(Purple) 	=66

29 > 32 %  MAGENTA / GREEN		F(Pur/Gree) 	=77

33 > 36 %  GREEN				F(Green) 	=87

37 > 40 %  GREEN / ORANGE			F(Gree/Oran) 	=96

41 > 44 %  ORANGE				F(Orange) 	=107

45 > 48 %  ORANGE / BLUE			F(Oran/Blue) 	=118

49 > 52 %  BLUE				F(Blue) 	=128

53 > 56 %  BLUE / PINK			F(Blue/Pink) 	=137

57 > 60 %  PINK				F(Pink) 		=147

61 > 64 %  PINK / WHITE			F(Pink/Whi) 	=158

65 > 69 %  SPIN  1				F(RotSpd1) 	=169

70 > 76 %  SPIN  2				F(RotSpd2) 	=189

77 > 81 %  SPIN  3				F(RotSpd3) 	=200

82 > 86 %  SPIN  4				F(RotSpd4) 	=214

87 > 91 %  SPIN  5				F(RotSpd5) 	=227

92 > 96 %  SPIN  6				F(RotSpd6) 	=239

97 > 100%  SPIN  7				F(RotSpd7) 	=255

OPTION : PERSONALITY DIP-SWITCH  N° ### in position ON ( from 0 to 60% ) give access

to the PROPORTIONAL control of the color wheel with the following options:

-- Possibility of choice of any needed colour or any portion between one colour and

the adjacent one

-- Possibility to syncronize the colour changement with the other functions of the

projector ( i.e. with the beam movement )

Note : Unchanged function from 61% to 100%

CHANNEL  2  =  EFFECTS WHEEL (COLOR CORRECTION)

percent values				decimal values

1  >  25  %    OPEN				F(Open) 	=0

26  > 50   %    3200°K				F(3200K) 	=97

51  > 75   %    6000°K				F(6000K) 	=160

CHANNEL  3  =  IRIS

percent values				decimal values

0   %  CLOSED				F(Closed)          =0

> <

100  %  OPEN					F(Open)	=255

CHANNEL  4  DIMMER / SHUTTER

percent values				decimal values

0  %   BLACK-OUT				F(Blk-Out)	=0

> <     INCREASING DIMMER LEVEL

50 >  53 %  DIMMER LEVEL 100% (OPEN)	F(FullyOpen)	=128

54 >  58 % STROBE EFFECT  Spd 1		F(StrobeSpd1)	=145

59 >  63 %  "     "     "     "     "     "  2		F(StrobeSpd2)	=160

64 >  69 %  "     "     "     "     "     "  3		F(StrobeSpd3)	=172

70 >  74 %  "     "     "     "     "     "  4		F(StrobeSpd4)	=184

75 >  79 %  "     "     "     "     "     "  5		F(StrobeSpd5)	=196

80 >  84 %  "     "     "     "     "     "  6		F(StrobeSpd6)	=210

85 >  89 %  "     "     "     "     "     "  7		F(StrobeSpd7)	=222

90 >  94 %  "     "     "     "     "     "  8		F(StrobeSpd8)	=234

95 > 100 %  DIMMER LEVEL 100% (OPEN)	F(FullyOpen)	=255

CHANNEL  5  FOCUSING  ( MOTORE IN CORRENTE CONTINUA)

percent values				decimal values

0  %   CLOSE FOCUS			F(CloseFocus)	=0

100 %   FOCUS TO INFINITE		F(FocusInfinit)	=255

LISTA DIP-SWITCHES CONTROL ASSIGNEMENT SECTION

10 	LAMP ON

9 	SELF TEST

8	LINEAR CONTROL COLOUR WHEEL

7

6

5	FOCUS

4	DIMMER

3	IRIS

2	COLOUR CORRECTION

1	COLOUR